Install ROS nmea_navsat_driver enhanced driver

- Log into the Tractor

- $ cd catkin_ws/src

- $ git clone https://github.com/ros-agriculture/nmea_navsat_driver.git

- $ cd ..

- From ~/catkin_ws, $ catkin_make

- $ source devel/setup.bash

- $ rospack find nmea_navsat_driver #  returns /home/al/catkin_ws/src/nmea_navsat_driver

- Launch sublime and open nmea_serial_driver.launch

- $ cd /home/tractor/catkin_ws/src/nmea_navsat_driver/launch

- Update the gps port and baud  input info

  - Open FileZilla

  - Go to /home/ubuntu/catkin_ws/src/nmea_navsat_driver/launch

  - Edit nmea_serial_driver.launch - a udev rule for “gps” needs to be in place

| From | To |
| --- | --- |
| <arg name="port" default="/dev/ttyUSB0" /> <arg name="baud" default="4800" /> | <arg name="port" default="/dev/gps" /> <arg name="baud" default="115200" /> |

- $ roslaunch nmea_serial_driver.launch

Trying the non-ROSAG version

- Log into the Tractor

- $ cd catkin_ws/src

- Delete the previous version

- Trying, $ git clone https://github.com/ros-drivers/nmea_navsat_driver.git

- $ cd ..

- From ~/catkin_ws, $ catkin_make

Failed -- Could NOT find catkin_virtualenv (missing: catkin_virtualenv_DIR) $ sudo apt-get install ros-noetic-catkin-virtualenv

#### Running command: "make -j2 -l2" in "/home/tractor/catkin_ws/build" #### Scanning dependencies of target nmea_navsat_driver_generate_virtualenv [ 25%] Generate virtualenv in /home/tractor/catkin_ws/build/venv True [ 50%] Install requirements to /home/tractor/catkin_ws/build/venv ERROR: launchpadlib 1.10.13 requires testresources, which is not installed. [ 75%] Prepare relocated virtualenvs for develspace and installspace [100%] Per-package virtualenv target [100%] Built target nmea_navsat_driver_generate_virtualenv tractor@tractor-HP-ProBook-645-G1:~/catkin_ws$ sudo apt install python3-pip

- $ source devel/setup.bash

- $ rospack find nmea_navsat_driver #  returns /home/al/catkin_ws/src/nmea_navsat_driver

- Launch sublime and open nmea_serial_driver.launch

- Make sure power is turned on to the GPS board and there are lights associated with receiving satellite signals

If you need to add a debug statement to print the nmea sentences Folder: /home/ubuntu/catkin_ws/src/nmea_navsat_driver/src/libnmea_navsat_driver Program: driver.py rospy.loginfo("nmea_string: %s", nmea_string) # added line 163 for debugging

- $ roslaunch nmea_serial_driver.launch

Comparing ROSAg and ROS

| ROS Ag https://github.com/ros-agriculture/nmea_navsat_driver/blob/master/src/libnmea_navsat_driver/driver.py | ROS https://github.com/ros-drivers/nmea_navsat_driver/blob/master/src/libnmea_navsat_driver/driver.py |
| --- | --- |
|  | nmea_navsat_driver/driver.py at master · ros-drivers/nmea_navsat_driver · GitHub |
| This is not committed Note: https://docs.ros.org/en/jade/api/sensor_msgs/html/msg/NavSatStatus.html int8 STATUS_SBAS_FIX = 1 # with satellite-based augmentation int8 STATUS_GBAS_FIX = 2 # with ground-based augmentation |  |

GPS Quality indicator:

0: Fix not valid

1: GPS fix

2: Differential GPS fix, OmniSTAR VBS

4: Real-Time Kinematic, fixed integers

5: Real-Time Kinematic, float integers, OmniSTAR XP/HP or Location RTK

This was in my startup procedures:

It suggests to me a “status” of 2 equates to a “good” RTK fix

You could try and use plotjuggler to plot the LAT/LON and see the drift

OLD NOTES FOR REFERENCE ONLY

Reference: https://github.com/ros-agriculture/nmea_navsat_driver

The base driver has been enhanced to publish speed and heading.

On Rpi:

- PuTTy into RPi $ plink RPI_on_ZeroTier -pw ubuntu

- $ cd catkin_ws/src

- $ git clone https://github.com/ros-agriculture/nmea_navsat_driver.git

- cd ..

- From ~/catkin_ws, $ catkin_make

- $ source devel/setup.bash

- $ rospack find nmea_navsat_driver #  returns /home/al/catkin_ws/src/nmea_navsat_driver

- $ cd /home/ubuntu/catkin_ws/src/nmea_navsat_driver/launch

- Update the gps port and baud  input info

  - Open FileZilla

  - Go to /home/ubuntu/catkin_ws/src/nmea_navsat_driver/launch

  - Edit nmea_serial_driver.launch

| From | To |
| --- | --- |
| <arg name="port" default="/dev/ttyUSB0" /> <arg name="baud" default="4800" /> | <arg name="port" default="/dev/gps" /> <arg name="baud" default="115200" /> |

- $ roslaunch nmea_serial_driver.launch

If you need to add a debug statement to print the nmea sentences

Folder: /home/ubuntu/catkin_ws/src/nmea_navsat_driver/src/libnmea_navsat_driver

Program: driver.py

rospy.loginfo("nmea_string: %s", nmea_string)                   # added line 163 for debugging

Check the GPGGA sentence after the longitude to see what the status is.  The example below is “0”; I had to start the RTK base station and move the tractor outdoors.

Afterwhich I am getting a 4 for status with 9 satellites.

$ rostopic echo /fix

Power up the base station sending correction data

Power up the tractor gps

$ roslaunch  /home/tractor/catkin_ws/src/nmea_navsat_driver/launch/nmea_serial_driver.launch

11/19/22

Gps-odom uses “import geonav_transform.geonav_conversions as gc”

https://get-help.robotigniteacademy.com/t/no-module-name-geonav-transform-geonav-conversions/14494/5

$ cd /home/tractor/catkin_ws/src $ git clone https://github.com/bsb808/geonav_transform.git $ cd /home/tractor/catkin_ws/ $ catkin_make $ source devel/setup.bash; rospack profile

Above did not produce errors, but gps_odom.py could not find library

$ python gazebo_elestero_ex.py

python3 -m pip install "SomeProject"

python3 -m pip install "geonav_transform"