Adjust yaml file and map

https://answers.ros.org/question/322919/to-understand-the-concept-of-origin-field-from-map_server/

Original:

#changes by sam #image: field.png image: ros_farm_v01.png #resolution: 0.1 resolution: .1 #origin: [-10.0, -10.0, 0.0] origin: [-30.0, -30.0, 0.0] negate: 0 occupied_thresh: 0.65 free_thresh: 0.196

origin : The 2-D pose of the lower-left pixel in the map, as (x, y, yaw), with yaw as counterclockwise rotation (yaw=0 means no rotation). Many parts of the system currently ignore yaw. link

the origin is up to you, and the resolution is the size in meters of a pixel of your image.

Capture an image from Google Maps

So lets say the image represents a lot that is 220.69 feet wide and the image is 566 pixels wide 67.266312/566=0.1188 Resolution = 0.1188

https://answers.ros.org/question/178972/about-resolution-on-yaml/

Fusion 360 Export to PNG https://forums.autodesk.com/t5/fusion-360-design-validate/export-to-png/td-p/5477372

Background on using satellite images in rvis:

- https://recordnotfound.com/rviz-satellite-gareth-cross-97256

- Construct video on mapping: https://www.youtube.com/watch?v=K1ZFkR4YsRQ

  - Mentioned in the video: http://wiki.ros.org/map_server along with map_saver

- Not a how to, but someone trying to create a self-driving golf cart: https://neilnie.com/2019/07/09/self-driving-golf-cart-autonomous-navigation-with-the-ros-navigation-stack-part-2-path-planning/

- https://github.com/gareth-cross/rviz_satellite

### rviz_satellite setup: - $ cd ~/catkin_ws/src - $ git clone https://github.com/gareth-cross/rviz_satellite.git - $ cd ~/catkin_ws/ - $ catkin build - $ source devel/setup.bash - $ rospack find rviz_satellite #returns /home/al/catkin_ws/src/rviz_satellite - $ cd /home/al/catkin_ws/src/rviz_satellite/launch - $ roslaunch demo.launch

- tool which converts KML/KMZ files into my custom path format: https://github.com/trainman419/dagny_robomagellan/blob/670a45856950bc4008bbe05aa838a7b1c70a9332/goal_list/scripts/kml2yaml.py

Trying to use openstreet for tiles

https://operations.osmfoundation.org/policies/tiles/

I have the same problem

https://www.latlong.net/c/?lat=39.951121&long=-75.189930

https://github.com/gareth-cross/rviz_satellite/issues/84

|  | origin: [-10.0, -10.0, 0.0] |
| --- | --- |
|  | origin: [-30.0, -8.0, 0.0] |